1)Colour of LED using SW1

#include <stdint.h>

#include <stdio.h>

#include "inc/tm4c123gh6pm.h"

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

int main(void)

{

/* enable clock to GPIOF at clock gating control register */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5; // Enable Port F Clock

/* set PF2, PF3, PF1 as output (LEDs) */

GPIO_PORTF_DIR_R = 0x0E;

/* set PF4 as input (SW1 button) */

GPIO_PORTF_DIR_R &= ~0x10;

/* enable pull-up resistor on PF4 */

GPIO_PORTF_PUR_R |= 0x10;

/* enable digital function on PF4, PF3, PF2, PF1 */

GPIO_PORTF_DEN_R = 0x1E;

/* LED color states array */

uint8_t led_colors[] = {0x02, 0x04, 0x08, 0x0E, 0x0C, 0x0A};

int led_index = 0;

while(1)

{

/* Wait for button press (PF4 LOW) */

while ((GPIO_PORTF_DATA_R & 0x10) != 0);  // Wait until pressed (PF4 = 0)

delayMs(20);  // debounce delay

/* Wait for button release */

while ((GPIO_PORTF_DATA_R & 0x10) == 0);  // Wait until released (PF4 = 1)

delayMs(20);  // debounce delay

/* Set LED to next color */

GPIO_PORTF_DATA_R = led_colors[led_index];

led_index = (led_index + 1) % (sizeof(led_colors) / sizeof(led_colors[0]));

}

}

2)Red color LED only when the sw1 is pressed

/*  RGB LED */

#include <stdint.h>

#include <stdio.h>

#include "inc/tm4c123gh6pm.h"

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

int main(void)

{

SYSCTL_RCGCGPIO_R |= 0x20;           // Enable clock to Port F

while ((SYSCTL_PRGPIO_R & SYSCTL_PRGPIO_R5) == 0) {};// Wait until Port F is ready

GPIO_PORTF_DIR_R |= 0x02;            // PF1 (Red LED) as output

GPIO_PORTF_DIR_R &= ~0x10;           // PF4 (SW1) as input

GPIO_PORTF_DEN_R |= 0x12;            // Enable digital on PF1 and PF4

GPIO_PORTF_PUR_R |= 0x10;            // Enable pull-up on PF4

while (1)

{

if ((GPIO_PORTF_DATA_R & 0x10) == 0) // If button is pressed (PF4 == 0)

GPIO_PORTF_DATA_R |= 0x02;       // Turn ON red LED (PF1)

else

GPIO_PORTF_DATA_R &= ~0x02;      // Turn OFF red LED

}

}

3)Blinking of LEDs

/*  RGB LED */

#include <stdint.h>

#include <stdio.h>

#include "inc/tm4c123gh6pm.h"

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

int main(void)

{

/* enable clock to GPIOF/B/C at clock gating control register */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5;//PF

/* enable the GPIO pins for the PF2/PF3/PB3/PC4 as output */

GPIO_PORTF_DIR_R = 0x0E;//PF2/3

/* enable the GPIO pins for digital function */

GPIO_PORTF_DEN_R = 0x0E;

while(1)

{

GPIO_PORTF_DATA_R = 0x02;

delayMs(100);

GPIO_PORTF_DATA_R = 0x04;

delayMs(500);

GPIO_PORTF_DATA_R = 0x08;

delayMs(500);

GPIO_PORTF_DATA_R = 0x0E;

delayMs(500);

GPIO_PORTF_DATA_R = 0x0C;

delayMs(500);

GPIO_PORTF_DATA_R = 0x0A;

delayMs(500);

}}

/* delay n milliseconds (16 MHz CPU clock) */

4)SSD Counter

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

void shift_out1(unsigned char str);

int main(void)

{

SYSCTL_RCGCGPIO_R = 0x10;     // Enable clock to GPIOE

GPIO_PORTE_DIR_R = 0x1F;      // PE0-PE4 as output (5 bits)

GPIO_PORTE_DEN_R = 0x1F;      // Digital enable for PE0-PE4

unsigned char digits[10] = {

0xFC,  // 0

0x60,  // 1

0xDA,  // 2

0xF2,  // 3

0x66,  // 4

0xB6,  // 5

0xBE,  // 6

0xE0,  // 7

0xFE,  // 8

0xF6   // 9

};

unsigned int count = 0;

while(1)

{

unsigned int thousands = count / 1000;

unsigned int hundreds = (count / 100) % 10;

unsigned int tens = (count / 10) % 10;

unsigned int units = count % 10;

shift_out1(digits[tens]);       // DS4 - units

shift_out1(digits[hundreds]);        // DS3 - tens

shift_out1(digits[thousands]);    // DS2 - hundreds

shift_out1(digits[units]);   // DS1 - thousands

delayMs(100);

count++;

if(count > 9999)

count = 0;

}

}

void shift_out1(unsigned char str)

{

unsigned char j = 0, check;

for(j = 0; j <= 7; j++)

{

GPIO_PORTE_DATA_R = 0x00;       // PE3 (SCLK) low

check = (str & (1 << j));       // Get bit j

if(check)

GPIO_PORTE_DATA_R = 0x04;   // PE2 (SDAT) high if bit is 1

else

GPIO_PORTE_DATA_R |= 0x00;  // PE2 low if bit is 0 (redundant)

GPIO_PORTE_DATA_R |= 0x08;      // PE3 (SCLK) high - rising clock edge

GPIO_PORTE_DATA_R |= 0x10;      // PE4 (RCLK) high - latch output

}

}

void delayMs(int n)

{

int i, j;

for(i = 0; i < n; i++)

for(j = 0; j < 4180; j++)

{

// Do nothing, just wait

}

}

5)ADC on SSD

/* potmeter 10k resistor at max , current flowing through resistor is 0.4095ma current flows. value

displayed on ssd id in mv */

#include "inc\tm4c123gh6pm.h"

#include <stdbool.h>

#include <stdint.h>

void delayMs(int n)

{

int i,j;

for(i=0;i<n;i++)

for(j=0;j<4180;j++)

{}

}

void shift_out1(unsigned char str );

int volatile int num1,num2,num3,num4;

int main(void)

{

volatile int result;

unsigned char a[16] ={0xFC,0x60,0xDA,0xF2,0x66,0xB6,0xBE,0xE0,0xFE,0xF6,0xEE,0x3E,0x9C,0x7A,0x9E,0x8E};

/* enable clocks */

SYSCTL_RCGCGPIO_R |= 0x08; /* enable clock to GPIOE (AIN0 is on PE3) */

SYSCTL_RCGCADC_R |= 1; /* enable clock to ADC0 */

SYSCTL_RCGCGPIO_R |= 0x10;

GPIO_PORTE_DIR_R |= 0x1F;

GPIO_PORTE_DEN_R |= 0x1F;

/* initialize PE3 for AIN0 input */

GPIO_PORTD_AFSEL_R |= 8; /* enable alternate function */

GPIO_PORTD_DEN_R &= ~8; /* disable digital function */

GPIO_PORTD_AMSEL_R |= 8; /* enable analog function */

//unsigned int i;

/* initialize ADC0 */

ADC0_ACTSS_R &= ~1; /* disable SS3 during configuration */

ADC0_EMUX_R &= ~0x000F; /* software trigger conversion */

ADC0_SSMUX0_R &= ~0xFFFFFFFF;

ADC0_SSMUX0_R |= 0x04; /* get input from channel 0 */

ADC0_SSCTL0_R |= 0x06; /* take one sample at a time, set flag at 1st sample */

ADC0_ACTSS_R |= 0x01; /* enable ADC0 sequencer 3 */

while(1)

{

ADC0_PSSI_R |= 1; /* start a conversion sequence 3 */

while((ADC0_RIS_R & 1) == 0); /* wait for conversion complete */

result = ADC0_SSFIFO0_R; /* read conversion result */

ADC0_ISC_R = 1; /* clear completion flag */

num1 = result%10;//copies data from the specified location

result = result/10;

num2 = result%10;

result = result/10;

num3 = result%10;

result = result/10;

num4 = result%10;

shift_out1(a[num1]);

shift_out1(a[num2]);

shift_out1(a[num3]);

shift_out1(a[num4]);

shift_out1(0x00);

delayMs(1000);

}

}

void shift_out1(unsigned char str)

{

unsigned char j=0,check;

for(j=0;j<=7;j++)

{

GPIO_PORTE_DATA_R = 0x00; //PE3 pin(sclk) is low (0000 0000)

check = (str &(1<<j));

if(check)

GPIO_PORTE_DATA_R = 0x04; //PE2 pin(sdat) is high (0000 0100)

else

GPIO_PORTE_DATA_R |= 0x00;

GPIO_PORTE_DATA_R |= 0x08; //PE3 pin(sclk) is high (0000 1000)

GPIO_PORTE_DATA_R |= 0x10;

}

}

6)DAC sinusoidal input

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

int main(void)

{

uint32_t duty;

const uint32_t load_val = 64000;  // PWM period

// Enable clocks to PWM1 and Port F

SYSCTL_RCGCPWM_R |= 0x02;         // PWM1 clock enable

SYSCTL_RCGCGPIO_R |= 0x20;        // Port F clock enable

// Configure PWM clock divider to 64

SYSCTL_RCC_R = (SYSCTL_RCC_R & ~0x000E0000) | 0x000E0000;  // PWM divider 64

// Configure PF3 for PWM output (M1PWM7)

GPIO_PORTF_AFSEL_R |= 0x08;        // Enable alternate function on PF3

GPIO_PORTF_PCTL_R = (GPIO_PORTF_PCTL_R & 0xFFFF0FFF) | 0x00005000;  // M1PWM7 on PF3

GPIO_PORTF_DEN_R |= 0x08;          // Digital enable PF3

// Configure PWM generator 3, generator B (M1PWM7)

PWM1_3_CTL_R = 0;                  // Disable generator 3 during setup

PWM1_3_GENB_R = 0x0000008C;        // Set PWM output behavior: output set on reload, clear on comparator match

PWM1_3_LOAD_R = load_val;           // Set PWM period

PWM1_3_CMPA_R = 0;                  // Start with 0% duty cycle

PWM1_3_CTL_R = 1;                   // Enable generator 3

PWM1_ENABLE_R |= 0x80;              // Enable PWM output on M1PWM7 (PF3)

while(1)

{

// Fade LED from low to high brightness

for (duty = 0; duty < load_val; duty += 256) {

PWM1_3_CMPA_R = duty;

delayMs(30);

}

// Fade LED from high to low brightness

for (duty = load_val; duty > 0; duty -= 256) {

PWM1_3_CMPA_R = duty;

delayMs(30);

}

}

}

void delayMs(int n)

{

volatile int i, j;

for (i = 0; i < n; i++)

for (j = 0; j < 3180; j++) {}  // Approximate 1 ms delay at 16 MHz

}

7)Buzzer

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

int value;

int main(void)

{

/* enable clock to GPIO F/D at clock gating control register *//* GPIO PORT D PIN 7 is

connected to Buzzer */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R3;//PD

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5;//PF

/* enable the GPIO pins for the PF2/PF3/PB3/PC4 as output */

GPIO_PORTD_LOCK_R = 0x4C4F434B; // unlock GPIO Port D

GPIO_PORTD_CR_R = 0x80; // allow changes to PD7-1 for GPIO

GPIO_PORTD_DIR_R = 0xf0;//PD7

GPIO_PORTD_DEN_R = 0xf0; /* enable the GPIO pins for digital function */

GPIO_PORTF_DIR_R = 0x08; /* set PORTF3 pin as output (LED) pin */

/* and PORTF4 as input, SW1 is on PORTF4 */

GPIO_PORTF_DEN_R = 0x18; /* set PORTF pins 4-3 as digital pins */

GPIO_PORTF_PUR_R = 0x10; /* enable pull up for pin 4 */

while (1)

{

value = GPIO_PORTF_DATA_R; /* read data from PORTF */

value = ~value; /* switch is low active; LED is high active */

value = value >> 1; /* shift it right to display on green LED */

GPIO_PORTF_DATA_R = value; /* put it on the green LED */

value = value << 5;

GPIO_PORTD_DATA_R = value;

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{} /* do nothing for 1 ms */

}

8)Stepper Motor

/*  p2_3b.c: Toggling LEDs using special function registers by their names defined in the TivaWare header file */

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

int main(void)

{

int i;  // loop variable

/* enable clock to GPIOF/B/C at clock gating control register */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5;

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R1;

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R2;

/* enable the GPIO pins for the PF2/PF3/PB3/PC4 as output */

GPIO_PORTF_DIR_R = 0x0E;  // PF1-3 as output (bits 1,2,3) - here 0x0E is bits 1,2,3

GPIO_PORTB_DIR_R = 0x08;  // PB3 as output

GPIO_PORTC_DIR_R = 0x10;  // PC4 as output

/* enable the GPIO pins for digital function */

GPIO_PORTF_DEN_R = 0x0E;

GPIO_PORTB_DEN_R = 0x08;

GPIO_PORTC_DEN_R = 0x10;

while(1)

{

for (i = 0; i < 50; i++)  // rotate stepper 100 times

{

// Step 1: PF2 ON

GPIO_PORTF_DATA_R = 0x04;

delayMs(5);

GPIO_PORTF_DATA_R = 0;

delayMs(5);

// Step 2: PF3 ON

GPIO_PORTF_DATA_R = 0x08;

delayMs(5);

GPIO_PORTF_DATA_R = 0;

delayMs(5);

// Step 3: PB3 ON

GPIO_PORTB_DATA_R = 0x08;

delayMs(5);

GPIO_PORTB_DATA_R = 0;

delayMs(5);

// Step 4: PC4 ON

GPIO_PORTC_DATA_R = 0x10;

delayMs(5);

GPIO_PORTC_DATA_R = 0;

delayMs(5);

}

// After 100 rotations, stop the motor by clearing outputs

GPIO_PORTF_DATA_R = 0;

GPIO_PORTB_DATA_R = 0;

GPIO_PORTC_DATA_R = 0;

while(1) {}  // Infinite loop to stop further execution

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

/* This function is called by the startup assembly code to perform system specific initialization tasks. */

9)Stepper Motor one full rotation

/*  p2_3b.c: Toggling LEDs using special function registers by their names defined in the TivaWare header file */

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

int main(void)

{

int i;  // loop variable

/* enable clock to GPIOF/B/C at clock gating control register */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5;

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R1;

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R2;

/* enable the GPIO pins for the PF2/PF3/PB3/PC4 as output */

GPIO_PORTF_DIR_R = 0x0E;

GPIO_PORTB_DIR_R = 0x08;

GPIO_PORTC_DIR_R = 0x10;

/* enable the GPIO pins for digital function */

GPIO_PORTF_DEN_R = 0x0E;

GPIO_PORTB_DEN_R = 0x08;

GPIO_PORTC_DEN_R = 0x10;

// Rotate stepper motor clockwise for 100 rotations

for (i = 0; i < 100; i++)

{

// Step 1: PC4 ON

GPIO_PORTC_DATA_R = 0x10;

delayMs(5);

GPIO_PORTC_DATA_R = 0;

delayMs(5);

// Step 2: PB3 ON

GPIO_PORTB_DATA_R = 0x08;

delayMs(5);

GPIO_PORTB_DATA_R = 0;

delayMs(5);

// Step 3: PF3 ON

GPIO_PORTF_DATA_R = 0x08;

delayMs(5);

GPIO_PORTF_DATA_R = 0;

delayMs(5);

// Step 4: PF2 ON

GPIO_PORTF_DATA_R = 0x04;

delayMs(5);

GPIO_PORTF_DATA_R = 0;

delayMs(5);

}

// Stop motor after 100 rotations by turning off all pins

GPIO_PORTF_DATA_R = 0;

GPIO_PORTB_DATA_R = 0;

GPIO_PORTC_DATA_R = 0;

// Infinite loop to stop program

while(1)

{

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

10)DC Motor

/*  DC_Motor _CLK*/

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

int main(void)

{

/* enable clock to GPIOF/B/C at clock gating control register */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5;//PF

/* enable the GPIO pins for the PF2/PF3/PB3/PC4 as output */

GPIO_PORTF_DIR_R = 0x0E;//PF2/3

/* enable the GPIO pins for digital function */

GPIO_PORTF_DEN_R = 0x0E;

while(1)

{

GPIO_PORTF_DATA_R = 0x04;

delayMs(500);

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

/* This function is called by the startup assembly code to perform system specific initialization tasks. */

11)LDR and Temperature Sensor

#include "inc\tm4c123gh6pm.h"

#include <stdbool.h>

#include <stdint.h>

void shift_out1(unsigned char str);

void delayMs(int n);

volatile int num1,num2,num3,num4;

unsigned int temp;

int main(void)

{  volatile int result;

unsigned char a[16] = {0xFC,0x60,0xDA,0xF2,0x66,0xB6,0xBE,0xE0,0xFE,0xF6,0xEE,0x3E,0x9C,0x7A,0x9E,0x8E};

// unsigned int i;

/* enable clocks */

SYSCTL_RCGCGPIO_R |= 0x08;   /* enable clock to GPIOE (AIN0 is on PE3) */

SYSCTL_RCGCADC_R |= 1;       /* enable clock to ADC0 */

SYSCTL_RCGCGPIO_R |= 0x10;

GPIO_PORTE_DIR_R |= 0x1F;

GPIO_PORTE_DEN_R |= 0x1F;

/* initialize PE3 for AIN0 input  */

GPIO_PORTD_AFSEL_R |= 1;       /* enable alternate function */

GPIO_PORTD_DEN_R &= ~1;        /* disable digital function */

GPIO_PORTD_AMSEL_R |= 1;       /* enable analog function */

/* initialize ADC0 */

ADC0_ACTSS_R &= ~1;        /* disable SS3 during configuration */

ADC0_EMUX_R &= ~0x000F;    /* software trigger conversion */

ADC0_SSMUX0_R &= ~0xFFFFFFFF;

ADC0_SSMUX0_R |= 0x07;         /* get input from channel 0 */

ADC0_SSCTL0_R |= 0x06;        /* take one sample at a time, set flag at 1st sample */

ADC0_ACTSS_R |= 0x01;           /* enable ADC0 sequencer 3 */

while(1)

{   ADC0_PSSI_R |= 1;        /* start a conversion sequence 3 */

while((ADC0_RIS_R & 1) == 0) ;   /* wait for conversion complete */

result = ADC0_SSFIFO0_R; /* read conversion result */

ADC0_ISC_R = 1;          /* clear completion flag */

temp= (1475 - (((2475 * result)) / 4096))/10;

temp= ((temp * 9) + 160) / 5;

result=temp;

num1 = result%10;//copies data from the specified location

result = result/10;

num2 = result%10;

result = result/10;

num3 = result%10;

result = result/10;

num4 = result%10;

shift_out1(a[num1]);

shift_out1(a[num2]);

shift_out1(a[num3]);

shift_out1(a[num4]);

shift_out1(0x00);

delayMs(200);

}}

void shift_out1(unsigned char str){

unsigned char j=0,check;

unsigned int i;

for(j=0;j<=7;j++){

GPIO_PORTE_DATA_R = 0x00;   //PE3 pin(sclk) is low (0000 0000)

check = (str &(1<<j));

if(check)

GPIO_PORTE_DATA_R =  0x04;  //PE2 pin(sdat) is high (0000 0100)

else

GPIO_PORTE_DATA_R |=  0x00;

GPIO_PORTE_DATA_R |=  0x08;  //PE3 pin(sclk) is high (0000 1000)

GPIO_PORTE_DATA_R |=  0x10;

}}

void delayMs(int n){

int i,j;

for(i=0;i<n;i++)

for(j=0;j<4180;j++)

{}

}

12)Hello World

#include <stdint.h>

#include <stdbool.h>

#include "inc\tm4c123gh6pm.h"

#include "driverlib/sysctl.h"

#define LCD_clear()   LCD_command(0x1)        /* Clear display LCD */

#define LCD_origin()  LCD_command(0x2)        /* Set to origin LCD */

#define LCD_row1()    LCD_command(0x80)        /* Begin at Line 1 */

#define LCD_row2()    LCD_command(0xC0)  /* Begin at Line 2 */

void shift_out1(unsigned char str);

void LCD_enable(void);

void LCD_command(unsigned char command);

void LCD_putc(unsigned char ascii);

void LCD_puts(unsigned char *lcd_string);

void LCD_init(void);

int main(void)

{

SYSCTL_RCGCGPIO_R |= 0x039;   /* enable clock to GPIOD/E/A/F */

/* initialize PE3 for AIN4 input  */

GPIO_PORTD_AFSEL_R |= 8;       /* enable alternate function */

GPIO_PORTD_DEN_R &= ~8;        /* disable digital function */

GPIO_PORTD_AMSEL_R |= 8;       /* enable analog function */

GPIO_PORTE_DIR_R |= 0x0C; /*   */

GPIO_PORTE_DEN_R |= 0x0C;

GPIO_PORTA_DIR_R |= 0x20;

GPIO_PORTA_DEN_R |= 0x20;

GPIO_PORTF_DIR_R |= 0x01;

GPIO_PORTF_DEN_R |= 0x01;

LCD_init(); //LCD 16x2 initializing

LCD_row1();

LCD_puts("     Hello ");

LCD_row2();

LCD_puts("     World ");

}

void LCD_puts(unsigned char *lcd_string)

{

while (*lcd_string)

{

LCD_putc(*lcd_string++);

}

}

void LCD_init()

{

SysCtlDelay(500000);

LCD_command(0x30);

SysCtlDelay(500000);

LCD_command(0x30);

SysCtlDelay(500000);

LCD_command(0x30);

SysCtlDelay(500000);

LCD_command(0x20);

SysCtlDelay(500000);

LCD_command(0x20);

SysCtlDelay(500000);

LCD_command(0x28); //Selecting 4bit LCD 16x2

SysCtlDelay(500000);

LCD_command(0x0C); //LCD cursor off

SysCtlDelay(500000);

LCD_command(0x06); //LCD display on

SysCtlDelay(500000);

LCD_command(0x01); //LCD clear screen

SysCtlDelay(500000);

}

unsigned char PP1 = 0x00;

void LCD_putc(unsigned char ascii)

{

unsigned char volatile  num;

num = ascii;

// here reversing the bits of lcd_data

//e.g 00110011 will become 110110

num = ((num & 0x11)<<3)| ((num & 0x22)<<1)|((num & 0x44)>>1)|((num & 0x88)>>3);

ascii = num;

//Sending higher nibble of lcd data

PP1 = (PP1 & 0xF0)|((ascii>>4) & 0x0F);

PP1 = PP1 | 0xA0; // setting 7th pin as high RS = 1, EN = 1;

PP1 = PP1 & ~(0x40); //RW = 0

//shifting data bits to shift register

shift_out1(PP1);

SysCtlDelay(500);

PP1 = PP1 & ~(0x20); // En = 0

shift_out1(PP1);

SysCtlDelay(500);

//Sending lower nibble of lcd data

PP1 = (PP1 & 0xF0)|(ascii & 0x0F);

PP1 = PP1 | 0xA0; // setting 7th bit as high RS = 1, EN = 1;

PP1 = PP1 & ~(0x40); //RW = 0

shift_out1(PP1);

SysCtlDelay(500);

PP1 = PP1 & ~(0x20); // EN = 0

shift_out1(PP1);

SysCtlDelay(500);

}

void shift_out1(unsigned char str)

{

unsigned char j=0,check;

GPIO_PORTA_DATA_R = 0x00;   //PA5 pin(stk) is low (0000 0000)

for(j=0;j<=7;j++)

{

GPIO_PORTE_DATA_R = 0x00;   //PE3 pin(sclk) is low (0000 0000)

check = (str &(1<<j));

if(check)

GPIO_PORTE_DATA_R |= 0x04;   //PE2 pin(sdat) is high (0000 0100)

else

GPIO_PORTE_DATA_R = 0x00;   //PE2 pin(sdat) is low (0000 0000)

GPIO_PORTE_DATA_R |= 0x08;   //PE3 pin(sclk) is high (0000 1000), sclk=1;

}

GPIO_PORTA_DATA_R |= 0x20;   //PA5 pin(stk) is high (0010 0000) , stk=1,storing completed;

}

unsigned char PP0 = 0x00;

void LCD_command(unsigned char command)

{

unsigned char volatile num;

num = command;

// here reversing the bits of lcd_command

//e.g 00110011 will become 11001100

num = ((num & 0x11)<<3)| ((num & 0x22)<<1)|((num & 0x44)>>1)|((num & 0x88)>>3);

command = num;

//Sending higher nibble of lcd data

PP0 = (PP0 & 0xF0)|((command>>4) & 0x0F);//ok

PP0 = PP0 & (~(3 << 7)); //  setting 7th pin as low RS = 0 , RW = 0

PP0 = PP0 | 0x20; // EN = 1

//shifting bits to shift register

shift_out1(PP0);

SysCtlDelay(500);

PP0 = PP0 & ~(0x20); //EN = 0

shift_out1(PP0);

SysCtlDelay(500);

//Sending lower nibble of lcd command

PP0 = (PP0 & 0xF0)|(command & 0x0F);

PP0 = PP0 & (~(3 << 7));   // RS = 0,RW = 0

PP0 = PP0 | 0x20; // EN = 1

//Sending lower nibble of lcd command

shift_out1(PP0);

SysCtlDelay(500);

PP0 = PP0 & ~(0x20); // EN = 0

shift_out1(PP0);

SysCtlDelay(500);}

13)Shifted lcd

while(1)

{

LCD_command(0x18); // Scroll display left by 1 character

//LCD_command(0x1C); // Scroll display right by 1 character

delay_ms(100);

}

14)Keypad

/* ************************************************

*Project name :Interfacing of analog hex keypad and Output displaying in LCD 16x2 using TIVA C launchpad

*Date : 12/04/2018

***************************************************/

#include <stdint.h>

#include <stdbool.h>

#include "inc\tm4c123gh6pm.h"

#include "driverlib/sysctl.h"

#define LCD_clear()   LCD_command(0x1)        /* Clear display LCD */

#define LCD_origin()  LCD_command(0x2)        /* Set to origin LCD */

#define LCD_row1()    LCD_command(0x80)        /* Begin at Line 1 */

#define LCD_row2()    LCD_command(0xC0)  /* Begin at Line 2 */

void shift_out1(unsigned char str);

void LCD_enable(void);

void LCD_command(unsigned char command);

void LCD_putc(unsigned char ascii);

void LCD_puts(unsigned char *lcd_string);

void LCD_init(void);

unsigned char key_scan(unsigned int volatile rec_val);//stores key pressed

int main(void)

{

uint32_t ui32ADC0Value;

unsigned char Dig_val;//values for LCD

volatile uint32_t ui32TempAvg,Temp_val;

SYSCTL_RCGCGPIO_R |= 0x039;   /* enable clock to GPIOD/E/A/F */

SYSCTL_RCGCADC_R |= 1;       /* enable clock to ADC0 */

/* initialize PE3 for AIN4 input  */

GPIO_PORTD_AFSEL_R |= 8;       /* enable alternate function */

GPIO_PORTD_DEN_R &= ~8;        /* disable digital function */

GPIO_PORTD_AMSEL_R |= 8;       /* enable analog function */

GPIO_PORTE_DIR_R |= 0x0C;			/*   */

GPIO_PORTE_DEN_R |= 0x0C;

GPIO_PORTA_DIR_R |= 0x20;

GPIO_PORTA_DEN_R |= 0x20;

GPIO_PORTF_DIR_R |= 0x01;

GPIO_PORTF_DEN_R |= 0x01;

/* initialize ADC0 */

ADC0_ACTSS_R &= ~1;        /* disable SS0 during configuration */

ADC0_EMUX_R &= ~0x000F;    /* software trigger conversion */

ADC0_SSMUX0_R |= 0x05  ;         /* get input from channel 4 */

ADC0_SSCTL0_R |= 0x06;        /* take one sample at a time, set flag at 1st sample */

ADC0_ACTSS_R |= 0x01;           /* enable ADC0 sequencer 0 */

LCD_init();	//LCD 16x2 initializing

LCD_row1();	//Cursor position at 1st line

LCD_puts("Interfacing");

LCD_row2();	//Cursor position at 2nd line

LCD_puts("Analog KEYPAD");

SysCtlDelay(5000000);

LCD_clear();

while(1)

{

ADC0_PSSI_R |= 1;        /* start a conversion sequence 0 */

while((ADC0_RIS_R & 1) == 0) ;   /* wait for conversion complete */

ui32ADC0Value = ADC0_SSFIFO0_R; /* read conversion result */

ADC0_ISC_R = 1;          /* clear completion flag */

Temp_val = ui32ADC0Value/10;

Dig_val = key_scan(Temp_val);//compare the key pressed

SysCtlDelay(50000);

LCD_row1();

LCD_puts("Press any Key");

LCD_row2();

if(Dig_val <= 'F')//if the ascii is less or equal to F put the value in LCD

LCD_putc(Dig_val);

}

}

unsigned char key_scan(unsigned int volatile rec_val)

{

if( rec_val == 373 || rec_val == 374)	return '0';	    // 0 key pressed

else if( rec_val == 376 || rec_val == 377 )  return '1'; 	  // 1st key pressed

else if( rec_val == 380 || rec_val == 379 || rec_val == 380)  return '2';	    // 2nd key pressed

else if( rec_val == 381 || rec_val == 382)  return '3'; 	  // 3rd key pressed

else if( rec_val == 357 || rec_val == 358 || rec_val == 359)  return '4';    	// 4th key pressed

else if( rec_val == 362 || rec_val == 363 || rec_val == 364)  return '5';    	// 5th key pressed

else if( rec_val == 367 || rec_val == 368 || rec_val == 369)  return '6';     // 6th key pressed

else if( rec_val == 371 || rec_val == 372)  return '7';   	// 7th key pressed

else if( rec_val == 317 || rec_val == 318 || rec_val == 319)  return '8';    	// 8th key pressed

else if( rec_val == 333 || rec_val == 334 || rec_val == 335)  return '9';   	// 9th key pressed

else if( rec_val == 345 || rec_val == 346 || rec_val == 347)  return 'A';   	// A  key pressed

else if( rec_val == 353 || rec_val == 354 || rec_val == 355)  return 'B';   	// B key pressed

else if( rec_val == 0)    return 'C';   	// C key pressed

else if( rec_val == 203 || rec_val == 204 || rec_val == 205)  return 'D';    	// D key pressed

else if( rec_val == 272 || rec_val == 273 || rec_val == 274)  return 'E';			// E key pressed

else if( rec_val == 306 || rec_val == 307 || rec_val == 308)  return 'F';   	// F key pressed

else

return 'G';

}

void LCD_puts(unsigned char *lcd_string)

{

while (*lcd_string)

{

LCD_putc(*lcd_string++);

}

}

void LCD_init()

{

SysCtlDelay(500000);

LCD_command(0x30);

SysCtlDelay(500000);

LCD_command(0x30);

SysCtlDelay(500000);

LCD_command(0x30);

SysCtlDelay(500000);

LCD_command(0x20);

SysCtlDelay(500000);

LCD_command(0x20);

SysCtlDelay(500000);

LCD_command(0x28); //Selecting 4bit LCD 16x2

SysCtlDelay(500000);

LCD_command(0x0C); //LCD cursor off

SysCtlDelay(500000);

LCD_command(0x06); //LCD display on

SysCtlDelay(500000);

LCD_command(0x01); //LCD clear screen

SysCtlDelay(500000);

}

unsigned char PP1 = 0x00;

void LCD_putc(unsigned char ascii)

{

unsigned char volatile  num;

num = ascii;

// here reversing the bits of lcd_data

//e.g 00110011 will become 110110

num = ((num & 0x11)<<3)| ((num & 0x22)<<1)|((num & 0x44)>>1)|((num & 0x88)>>3);

ascii = num;

//Sending higher nibble of lcd data

PP1 = (PP1 & 0xF0)|((ascii>>4) & 0x0F);

PP1 = PP1 | 0xA0; // setting 7th pin as high RS = 1, EN = 1;

PP1 = PP1 & ~(0x40); //RW = 0

//shifting data bits to shift register

shift_out1(PP1);

SysCtlDelay(500);

PP1 = PP1 & ~(0x20); // En = 0

shift_out1(PP1);

SysCtlDelay(500);

//Sending lower nibble of lcd data

PP1 = (PP1 & 0xF0)|(ascii & 0x0F);

PP1 = PP1 | 0xA0; // setting 7th bit as high RS = 1, EN = 1;

PP1 = PP1 & ~(0x40); //RW = 0

shift_out1(PP1);

SysCtlDelay(500);

PP1 = PP1 & ~(0x20); // EN = 0

shift_out1(PP1);

SysCtlDelay(500);

}

void shift_out1(unsigned char str)

{

unsigned char j=0,check;

GPIO_PORTA_DATA_R = 0x00;   //PA5 pin(stk) is low (0000 0000)

for(j=0;j<=7;j++)

{

GPIO_PORTE_DATA_R = 0x00;   //PE3 pin(sclk) is low (0000 0000)

check = (str &(1<<j));

if(check)

GPIO_PORTE_DATA_R |= 0x04;   //PE2 pin(sdat) is high (0000 0100)

else

GPIO_PORTE_DATA_R = 0x00;   //PE2 pin(sdat) is low (0000 0000)

GPIO_PORTE_DATA_R |= 0x08;   //PE3 pin(sclk) is high (0000 1000), sclk=1;

}

GPIO_PORTA_DATA_R |= 0x20;   //PA5 pin(stk) is high (0010 0000) , stk=1,storing completed;

}

unsigned char PP0 = 0x00;

void LCD_command(unsigned char command)

{

unsigned char volatile num;

num = command;

// here reversing the bits of lcd_command

//e.g 00110011 will become 11001100

num = ((num & 0x11)<<3)| ((num & 0x22)<<1)|((num & 0x44)>>1)|((num & 0x88)>>3);

command = num;

//Sending higher nibble of lcd data

PP0 = (PP0 & 0xF0)|((command>>4) & 0x0F);//ok

PP0 = PP0 & (~(3 << 7)); //  setting 7th pin as low RS = 0 , RW = 0

PP0 = PP0 | 0x20;	// EN = 1

//shifting bits to shift register

shift_out1(PP0);

SysCtlDelay(500);

PP0 = PP0 & ~(0x20); //EN = 0

shift_out1(PP0);

SysCtlDelay(500);

//Sending lower nibble of lcd command

PP0 = (PP0 & 0xF0)|(command & 0x0F);

PP0 = PP0 & (~(3 << 7));   // RS = 0,RW = 0

PP0 = PP0 | 0x20;	// EN = 1

//Sending lower nibble of lcd command

shift_out1(PP0);

SysCtlDelay(500);

PP0 = PP0 & ~(0x20); // EN = 0

shift_out1(PP0);

SysCtlDelay(500);

}

15)UART

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void UART0Tx(char c);

void delayMs(int n);

int main(void)

{

SYSCTL_RCGCUART_R = 1;  /* provide clock to UART0 */

SYSCTL_RCGCGPIO_R = 1;  /* enable clock to PORTB */

/* UART0 initialization */

UART0_CTL_R = 0;         /* disable UART0 */

UART0_IBRD_R = 104;      /* 16MHz/16=1MHz, 1MHz/104=9600 baud rate */

UART0_FBRD_R = 11;       /* fraction part*/

UART0_CC_R = 0;          /* use system clock */

UART0_LCRH_R = 0x60;     /* 8-bit, no parity, 1-stop bit, no FIFO */

UART0_CTL_R = 0x301;     /* enable UART0, TXE, RXE */

/* UART0 TX0 and RX0 use PB0 and PB1. Set them up. */

GPIO_PORTA_DEN_R = 0x03;      /* Make PB0 and PB1 as digital */

GPIO_PORTA_AFSEL_R = 0x03;    /* Use PB0,PB1 alternate function */

GPIO_PORTA_PCTL_R = 0x11;     /* configure PB0 and PB1 for UART */

delayMs(1);             /* wait for output line to stabilize */

UART0Tx('B');

UART0Tx('0');

UART0Tx('2');

UART0Tx(' ');

UART0Tx('S');

UART0Tx('r');

UART0Tx('i');

UART0Tx('k');

UART0Tx('a');

UART0Tx('r');

UART0Tx('B');

UART0Tx('0');

UART0Tx('3');

UART0Tx(' ');

UART0Tx('Y');

UART0Tx('a');

UART0Tx('s');

UART0Tx('h');

UART0Tx('w');

UART0Tx('a');

UART0Tx('n');

UART0Tx('t');

UART0Tx('h');

}

/* UART0 Transmit */

/* This function waits until the transmit buffer is available then */

/* writes the character in the transmit buffer. It does not wait */

void UART0Tx(char c)

{

while((UART0_FR_R & 0x20) != 0); /* wait until Tx buffer not full */

UART0_DR_R = c;                  /* before giving it another byte */

}

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

16)DC motor using pwm

#include <stdint.h>

#include "inc\tm4c123gh6pm.h"

void delayMs(int n);

int main(void)

{

/* Enable clock to GPIOF at clock gating control register */

SYSCTL_RCGCGPIO_R |= SYSCTL_RCGCGPIO_R5;  // PF

/* Configure PF2 and PF3 as outputs to control motor direction */

GPIO_PORTF_DIR_R = 0x0E;  // PF2 and PF3 as output (Motor control)

/* Enable the GPIO pins for digital function */

GPIO_PORTF_DEN_R = 0x0E;  // Enable digital on PF2 and PF3

// Set initial motor direction (Anti-clockwise, PF2 = 1, PF3 = 0)

GPIO_PORTF_DATA_R = 0x08;

// Set the on-time and off-time in milliseconds

uint32_t onTime = 250;   // Motor on time (in ms)

uint32_t offTime = 750;  // Motor off time (in ms)

while (1)

{

// Turn on the motor (PF2 = 1, PF3 = 0)

GPIO_PORTF_DATA_R = 0x08;

// Wait for the motor to run for the "on time"

delayMs(onTime);

// Turn off the motor (PF2 = 0, PF3 = 0)

GPIO_PORTF_DATA_R = 0x00;

// Wait for the "off time"

delayMs(offTime);

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for (i = 0; i < n; i++)

for (j = 0; j < 4180; j++)

{}  /* do nothing for 1 ms */

}

17)